# Automated Transpilation and Syndrome Decoding Pipeline for Silicon-Spin Quantum Error Correction: Bridging Qiskit, Stim, and PyMatching

## Executive Summary and Equal1 Hardware Context

Quantum hardware engineering at Equal1—spanning its silicon fabrication, cryo-electronics, and software engineering laboratories in Dublin, Delft, Sherbrooke, Fremont, and Timișoara—centers on integrating silicon spin qubits, cryogenic control logic, and real-time error correction onto a single commercial CMOS die1. Equal1's rack-mounted quantum computing systems, including the 6-qubit Bell-1 Quantum Server deployed to the European Space Agency and the RacQ data-center platform, are powered by the UnityQ Quantum System-on-Chip (SoC) fabricated on GlobalFoundries' 22FDX (22-nm fully depleted silicon-on-insulator) process1. Operating at approximately 300 millikelvin (0.3 K), the UnityQ architecture co-locates electron spin quantum dot arrays alongside on-die analog radio-frequency (RF) control, cycle-accurate digital timing logic, and embedded ARM Cortex processor cores dedicated to low-latency, sub-cycle quantum error correction (QEC)3.

Developing, validating, and benchmarking fault-tolerant quantum circuits for this hardware stack currently suffers from severe software fragmentation across three distinct open-source ecosystems: Qiskit, Stim, and PyMatching8. Quantum algorithm designers and calibration engineers construct logical and physical circuits in Qiskit using matrix-based gate representations and custom hardware-calibrated unitaries9. To evaluate fault-tolerance thresholds and multi-round syndrome extraction under realistic noise, researchers must translate these circuits into Stim, a high-performance stabilizer tableau simulator12. Finally, to test error-decoding performance and estimate logical error rates (LER), the resulting syndrome data and noise models must be converted into matching graphs for PyMatching, a Minimum-Weight Perfect Matching (MWPM) decoder12.

Because each tool operates on a fundamentally different mathematical abstraction—complex unitary statevectors in Qiskit, binary symplectic stabilizer tableaus and relative measurement records in Stim, and weighted Tanner graphs in PyMatching—engineers are currently forced to manually rewrite quantum circuits and custom gate definitions across all three notations13. Manual reconstruction introduces subtle phase-kickback bugs, qubit-endianness inversions, and classical register indexing mismatches while slowing down hardware-software co-design iterations17. Establishing an automated, mathematically rigorous transpilation and decoding pipeline resolves this bottleneck and provides a unified compiler frontend for silicon-spin fault-tolerant architectures3.

## Architectural Analysis of the Three-Tool Ecosystem

### Physical-to-Logical Co-Design Drivers in Silicon Spin Qubits

The necessity of bridging Qiskit, Stim, and PyMatching is directly dictated by the physical characteristics of CMOS silicon spin qubits3. On the UnityQ processor, single-qubit gates driven via electron spin resonance or baseband pulses achieve an average fidelity of 99.9% with a duration of 140 ns, while two-qubit gates mediated by voltage-pulsed Heisenberg exchange coupling across barrier gates between adjacent quantum dots achieve an average fidelity of 99.3% in 200 ns3. Parity readout via spin-to-charge conversion achieves 99.0% fidelity but requires 10 s (10,000 ns)—making measurement 50 to 70 times slower than logic gate execution and far slower than typical 500 ns superconducting readout windows3.

Because readout dominates the QEC cycle duration on silicon-spin hardware and charge noise or conduction-band valley-splitting variations introduce spatially inhomogeneous error rates across a 300-mm wafer, engineers must simulate hundreds of thousands of syndrome extraction rounds across varying code distances to tune the on-die decoder before tape-out or cryogenic deployment21. Each of the three tools in the laboratory workflow addresses a distinct layer of this problem, yet their internal representations remain mutually incompatible without a dedicated compiler bridge8.

| Architectural Dimension | Qiskit (Design & Unitary Layer) | Stim (Stabilizer & Fault Simulation Layer) | PyMatching (Syndrome Decoding Layer) |
| --- | --- | --- | --- |
| Primary Role in Workflow | Physical/logical circuit construction, custom unitary gate definition, pulse/transpiler synthesis9. | High-speed Monte Carlo sampling of noisy stabilizer circuits and extraction of Detector Error Models (DEM)12. | Minimum-Weight Perfect Matching (MWPM) syndrome decoding and Logical Error Rate (LER) benchmarking12. |
| Core Mathematical State | Complex statevector or density matrix ; gates as matrices 13. | Binary symplectic Stabilizer Tableau of stabilizers and destabilizers over ( bits + signs)13. | Weighted undirected decoding graph (Tanner graph) or sparse binary check matrix 14. |
| Memory & Complexity Scaling | space ( bytes); exponential wall time (~1 s per codeword at )13. | space (~6 MB at ); per gate, per random measurement13. | graph storage; near-linear average C++ sparse-blossom batch decoding (decode_batch)14. |
| Qubit & Classical Addressing | Named QuantumRegister and overwritable ClassicalRegister objects (qreg[i], creg[j])17. | Flat integer qubit indices (0, 1, ...) and an append-only global measurement record (rec[-1], rec[-2])12. | Detector node indices (D0, D1, ...), boundary nodes, and logical observable fault IDs (L0, L1, ...)12. |
| Gate & Error Constraints | Universal: accepts arbitrary unitaries, parameterized rotations , gates, and custom Gate subclasses13. | Strictly Clifford group (), resets, Pauli measurements (M, MPP), and Pauli noise channels13. | Strictly graphlike error mechanisms (each fault column in or DEM must flip detectors)14. |
| Noise Representation | External NoiseModel objects attached to simulators (qiskit_aer) or custom channel instructions12. | Explicit inline circuit instructions (DEPOLARIZE1, DEPOLARIZE2, X_ERROR, PAULI_CHANNEL_1/2)12. | Edge weights discretized into 24-bit even integers26. |

### Qiskit: Unitary Matrix Formalism and Circuit Construction

In the initial stage of the laboratory workflow, engineers use Qiskit (qiskit.circuit.QuantumCircuit) to express quantum algorithms, state preparation routines, and syndrome extraction blocks17. Qiskit models every quantum logic gate as an explicit  unitary matrix  satisfying , where consecutive gates act via left-multiplication in reverse chronological order on a statevector of  complex amplitudes10. While this formalism provides complete universality—allowing engineers to inject exact unitary matrices derived from finite-element Schrödinger-Poisson simulations of silicon-germanium (Si/SiGe) heterostructures or 22FDX FD-SOI quantum dot potentials5—it hits an immediate exponential wall when simulating fault-tolerant circuits13. Simulating even small distance-3 codes such as the , , or  stabilizer codes in Qiskit requires roughly 1 second per codeword, rendering million-shot threshold sweeps impossible18. Furthermore, Qiskit's built-in Extended Stabilizer simulator is restricted to CPU execution and caps out at 63 qubits40.

### Stim: Stabilizer Tableau Formalism and Detector Error Models

To overcome statevector scaling limits, the second stage relies on Stim, a vectorized C++ stabilizer simulator governed by the Gottesman-Knill theorem13. Rather than storing state amplitudes, Stim represents an -qubit quantum state by the  independent commuting Pauli generators () that stabilize it (), alongside  destabilizer generators tracked in an inverse Stabilizer Tableau 13. Because each Pauli operator on  qubits requires  bits plus a sign bit, storage scales quadratically at roughly  bits—meaning a 5,000-qubit state occupies roughly 6 MB and samples in 0.56 seconds, compared to  bytes for a statevector13. Applying any Clifford gate updates the tableau by conjugating the generators in  time, while folding a measurement gate on qubit  checks whether the  column in  contains anti-commuting  or  terms, resolving random outcomes via pivot elimination in  time or reporting deterministic outcomes directly from column signs13.

Crucially, Stim is not merely a circuit simulator; it functions as a space-time fault analysis engine12. To evaluate a QEC circuit in Stim, the translated circuit must incorporate three structural mechanisms absent from standard Qiskit circuits. First, because Stim does not accept external Qiskit-style NoiseModel wrappers12, hardware decoherence, gate infidelities, and readout bit-flips must be injected directly into the instruction stream as inline Pauli channels such as DEPOLARIZE1(p), DEPOLARIZE2(p), X_ERROR(p), Z_ERROR(p), PAULI_CHANNEL_1(px, py, pz), or PAULI_CHANNEL_2(...)12. Second, whereas Qiskit writes measurement outcomes into named, statically indexed classical bits (creg[0]) that can be overwritten across QEC cycles17, Stim has no named classical registers; every measurement instruction (M, MX, MY, MR, MPP) appends a boolean bit to a flat, monotonically growing history array accessed backwards from the current point in time via negative relative offsets (rec[-1] for the most recent measurement, rec[-2] for the prior measurement, and so on)12. Third, because Stim circuits have no built-in concept of a quantum code41, the circuit must explicitly declare DETECTOR(x, y, t) rec[-a] rec[-b] annotations, each specifying a set of measurement record offsets whose bitwise XOR parity is strictly deterministic in a noiseless run (typically comparing an ancilla stabilizer measurement in round  against the same ancilla in round )12. Similarly, OBSERVABLE_INCLUDE(L_idx) rec[-c] ... declares which physical data measurements combine to form the protected logical eigenvalue17.

Once annotated with noise, detectors, and logical observables, calling circuit.detector_error_model(decompose_errors=True) traces every elementary fault backward and forward through the stabilizer tableau over , outputting a Detector Error Model (stim.DetectorErrorModel or DEM)14. Each instruction in the DEM (for example, error(0.00193118) D0 D2 L0) specifies the consolidated probability that an independent physical fault flips a specific subset of detectors (D0, D2) and logical observables (L0)12.

### PyMatching: Tanner Graph Construction and Blossom Decoding

The third tool, PyMatching, consumes the syndrome data and the DEM generated by Stim to test whether the QEC protocol can successfully identify and correct hardware faults12. PyMatching implements the sparse blossom algorithm in C++ to solve the Minimum-Weight Perfect Matching problem on a decoding graph , which represents a graphlike Tanner graph of the circuit-level noise model14. In this graph, each detector  corresponds to a vertex , and each independent error mechanism flipping two detectors  and  forms an edge , while errors flipping a single detector  form a half-edge connected to a virtual boundary node14. When multiple physical faults produce the identical detector signature with probabilities  and , PyMatching merges parallel edges using the independent XOR probability combination rule:

PyMatching then assigns each edge a log-likelihood weight:

which is internally rescaled and discretized into 24-bit even integers up to  so that fast integer arithmetic can be used during blossom matching26.

Given a batch of binary syndrome vectors  sampled by Stim (compile_detector_sampler().sample(shots=N, separate_observables=True)), calling matching.decode_batch(s) executes entirely in C++ to find a minimum-weight edge set whose boundary matches the triggered detectors (), directly outputting the predicted logical observable flips14. Comparing these predictions against Stim's ground-truth actual_observables yields the logical error rate of the circuit12.

## Core Technical Bottlenecks and the Custom Gate Problem

Building an automated converter across Qiskit, Stim, and PyMatching requires solving four deep structural incompatibilities that cause naive dictionary-based translators to fail17.

### The Custom Quantum Gate Dichotomy: Clifford vs. Non-Clifford Matrices

The most critical requirement identified by the Equal1 laboratory team is the automatic conversion of user-defined custom quantum gates. In Qiskit, engineers create custom gates in three ways: as hierarchical composite sub-circuits (Gate subclasses with an internal .definition circuit), as raw  complex unitary matrices (UnitaryGate(U)), or as opaque timing and calibration markers (idle_#)9. Because Stim only executes stabilizer operations, the conversion tool cannot simply map gate names via a static lookup dictionary—the exact limitation that causes qiskit-qec's get_stim_circuits utility to throw an Unexpected operations exception whenever an unlisted gate appears17. Instead, the transpiler must implement a multi-stage mathematical resolution engine covering composite block consolidation, matrix-to-tableau synthesis, and non-Clifford routing.

| Custom Gate Category in Qiskit | Mathematical Structure | Automated Resolution Mechanism | Complexity / Output Target |
| --- | --- | --- | --- |
| Composite Sub-Circuit Gate (Gate with .definition) | Hierarchical DAG of elementary Clifford or rotation gates25. | Recursive unrolling via Qiskit HighLevelSynthesis pass while preserving gate boundary metadata via Stim instruction tags (inst[custom_tag])25. | unrolling into native stim.Circuit instructions25. |
| Exchange Pulse / Split-Clifford Sequence (e.g., paired ) | Individually non-Clifford matrices () that multiply into a Clifford27. | Pre-synthesis two-qubit unitary block consolidation (fuse(max_qubits=2)) multiplies adjacent matrices into a net Clifford prior to verification10. | per matrix multiplication; avoids false non-Clifford rejection27. |
| Arbitrary Clifford Unitary Matrix (UnitaryGate(U) with ) | matrix with entry norms , non-zero entries per row/col, and 13. | Fast entry/phase screening followed by stim.Tableau.from_unitary_matrix(U, endian="little") and tableau.to_circuit(method="elimination")19. | tableau extraction; elimination yielding Stim gates28. |
| Small Coherent Over-Rotation / Calibration Error () | Near-Clifford unitary representing physical control drift or crosstalk17. | Factor out ideal Clifford and apply Pauli twirling to to emit an inline PAULI_CHANNEL_1 or PAULI_CHANNEL_2 instruction17. | Exact preservation of syndrome Tanner graph topology in stim.DetectorErrorModel14. |
| Genuinely Non-Clifford Gate (, generic , Toffoli) | Unitary outside the normalizer of the Pauli group ()13. | Decompose into Clifford+/ and route to Stim-compatible near-Clifford simulators Tsim (GPU ZX-calculus) or Clifft40. | Exact detector sampling for 80+ qubits without statevector blowup40. |

In silicon spin qubits, the natural physical two-qubit interaction is the Heisenberg exchange pulse, which generates the  unitary22:

Individually,  is non-Clifford: conjugating the Pauli operator  by  produces a linear combination of Pauli matrices rather than a single Pauli product, meaning an isolated  cannot be represented in a Stim tableau13. However, hardware engineers routinely combine two  pulses interleaved with single-qubit ESR rotations to construct a standard Clifford CX, CZ, or iSWAP gate22. If the converter inspects custom gates one by one, it will falsely reject the circuit at the first  gate13. By running a two-qubit unitary block consolidation pass (analogous to Qiskit's ConsolidateBlocks or Qibo's fuse(max_qubits=2)) prior to Clifford verification, adjacent custom single- and two-qubit matrices are multiplied in reverse application order into a single composite  unitary  that is strictly Clifford10.

When the transpiler encounters an arbitrary consolidated -qubit unitary matrix , it must automatically verify whether  belongs to the Clifford group  and decompose it into native Stim instructions28. First, the engine performs a fast structural pre-screen: for any Clifford unitary , there exists a non-negative integer  such that every non-zero entry has magnitude , every row and column contains exactly  non-zero entries, and after dividing out the phase  of the first non-zero entry , every entry of  is purely real or purely imaginary within machine precision28. Matrices passing this  scan are passed to stim.Tableau.from_unitary_matrix(U, endian="little") (or Qiskit's Clifford.from_matrix(U) and CliffordUnitarySynthesis plugin)9. Internally, Stim's algorithm solves the synthesis problem in  time: it treats the first column of  as a stabilizer state , finds a circuit  using X, CNOT, and the 24 single-qubit Clifford gates that maps  by halving the number of non-zero amplitudes in each of  steps, and computes 28. If  is Clifford,  reduces to a phased permutation matrix; the algorithm identifies the unique set of CNOT gates that moves the non-zero entry of the last column to the bottom to yield a diagonal matrix , and then solves for the S gates from the phases of the -th entries of 28. Once the stim.Tableau object is extracted, calling tableau.to_circuit(method="elimination") (or method="graph_state") synthesizes a valid sequence of elementary Stim Clifford gates in  time with at most  gates, which is spliced directly into the target stim.Circuit19.

If a user defines a custom gate whose consolidated matrix is genuinely non-Clifford—causing stim.Tableau.from_unitary_matrix to raise a ValueError57—pure Stim tableau simulation is mathematically excluded by the Gottesman-Knill theorem13. The automated tool handles this through two configurable policies. When exact state fidelity of circuits containing localized non-Clifford gates (such as magic-state injection or small angle rotations) is required, the converter emits an extended Stim-compatible text format and routes execution to Tsim (tsim.Circuit, which compiles Stim circuits containing T and U3 instructions alongside DETECTOR and OBSERVABLE_INCLUDE via GPU-accelerated ZX-calculus stabilizer rank decomposition for 80+ qubits)40 or Clifft (an exact near-Clifford simulator that accepts Stim circuits with non-Clifford extensions and compiles a symbolic-coordinate sampling plan)52. Conversely, when the goal is configuring PyMatching for QEC threshold estimation under coherent calibration drift, small over-rotations are factored and Pauli-twirled into incoherent PAULI_CHANNEL_1 or PAULI_CHANNEL_2 noise instructions, preserving exact decoding graph compatibility17.

### Endianness, Phase Kickback, and Classical Register Translation

Even when all gates are Clifford, manual and semi-automated conversions frequently corrupt QEC syndrome statistics due to three subtle semantic discrepancies between Qiskit and Stim. First, Qiskit orders multi-qubit statevectors and unitary matrices in little-endian convention (where qubit 0 is the least significant bit on the right of the tensor product), whereas standard textbook definitions and default calls to stim.Tableau.from_unitary_matrix assume big-endian ordering unless endian="little" is explicitly specified10. Failing to enforce endian="little" reverses control and target qubits on asymmetric custom two-qubit gates.

Second, when synthesizing stabilizer measurement circuits in Qiskit, engineers frequently decompose a controlled- stabilizer check into a CX gate followed by a CZ gate18. Because , applying CX followed by CZ from an ancilla control qubit onto a data target qubit creates a phase kickback of  onto the control qubit, which randomizes stabilizer measurements unless paired mistakes accidentally cancel out18. The converter's peephole optimization pass must automatically detect adjacent CX(c, t) and CZ(c, t) pairs sharing control and target qubits and rewrite them into a native CY c t instruction with appropriate S-gate phase tracking18.

Third, in Qiskit, a multi-round syndrome extraction loop repeatedly measures  ancilla qubits into the same -bit ClassicalRegister (creg[0..m-1])17. In Stim, every measurement pushes a new entry onto the append-only global record stack17. The converter must maintain a sequential measurement_data history list recording the [register_name, index] pair for every measurement encountered17. Whenever a classical bit is referenced by a conditional feedback gate (inst.condition == 1) or a detector specification, the converter resolves the classical bit to its negative relative record target (StimTarget_rec) using the exact offset relation17:

### Hyperedge Decomposition and Non-Graphlike Error Mechanisms

When connecting Stim to PyMatching, invoking pymatching.Matching.from_stim_circuit(circuit) or from_detector_error_model(dem) fails whenever the circuit generates hyperedges—single physical error mechanisms that flip three or more detectors simultaneously58. Hyperedges arise naturally from  depolarizing errors in CSS surface codes (which trigger two -stabilizer detectors and two -stabilizer detectors, such as D1 D2 D3 D4), hook errors on syndrome ancillas, or non-CSS and hypergraph-product codes36.

Because PyMatching's blossom algorithm strictly requires graphlike error mechanisms where every column in the parity-check matrix  has weight 1 or 214, the conversion pipeline must invoke circuit.detector_error_model(decompose_errors=True)14. Under this flag, Stim searches for pieces of a large error occurring elsewhere as single graphlike errors and decomposes D1 D2 D3 D4 into D1 D3 ^ D2 D4 (decomposing  into ), allowing PyMatching to treat each component as an independent edge probability update36. However, if a user's custom gate or non-CSS stabilizer code produces an irreducible hyperedge that cannot be factored into existing graphlike edges, Stim raises a failed to decompose hyperedges into edges error59. A production-grade tool must catch this exception automatically, extract the full hypergraph check matrix via detector_error_model_to_check_matrices(dem) from the beliefmatching package, and either fall back to decompose_errors=False with a hypergraph-capable decoder (BeliefMatching or BpOsdDecoder) or alert the engineer to the exact fault source using circuit.explain_detector_error_model_errors()45.

## Phase-Based Engineering Plan for the Unified Conversion Tool

To deliver a maintainable, high-performance software package for the Equal1 Timișoara engineering team, development should be structured as a modular Python library built around a canonical Intermediate Representation (IR) and executed across five sequential engineering phases over a 16-week schedule.

### Phase 1: Canonical Intermediate Representation and Base Clifford Transpiler (Weeks 1–3)

The objective of Phase 1 is to eliminate manual circuit rewriting for all standard Clifford operations, measurements, resets, and classical control structures by translating Qiskit's Directed Acyclic Graph (DAGCircuit) into a structured stim.Circuit17. During input normalization, the transpiler inspects QuantumCircuit.qregs and QuantumCircuit.cregs to compute deterministic register offsets (qreg_offset, creg_offset) that map named quantum and classical registers onto flat global integer indices17. When physical layout coordinates exist on the Qiskit backend target—such as a 2D silicon quantum-dot array or linear chain21—the transpiler emits QUBIT_COORDS(x, y) q_idx headers at the start of the stim.Circuit to preserve spatial geometry for space-time decoding graphs12.

As the transpiler traverses the Qiskit DAG in topological order, it maps standard Clifford gates (id, x, y, z, h, s, sdg, sx, sxdg, cx, cy, cz, swap, iswap), resets (reset), measurements (measure), and structural barriers (barrier) to their native Stim counterparts while tracking measurement history in measurement_data17. Classically conditioned Pauli operations in Qiskit (inst.condition == 1) are translated into Stim feedback operations (CX, CY, CZ) targeting the relative classical record offset StimTarget_rec and destination qubit17. Because Stim's architecture assumes that mid-circuit QEC corrections are tracked by the classical control system's Pauli frame rather than applied via physical microwave pulses, the transpiler also provides an optional pass to strip in-circuit feedback and delegate corrections to the decoder34. Finally, rather than unrolling multi-round QEC experiments into thousands of flat instructions, the Phase 1 engine converts Qiskit ForLoopOp blocks or barrier-delimited syndrome rounds into native Stim REPEAT N { ... } blocks paired with SHIFT_COORDS(0, 0, 1) instructions, enabling Stim's loop-folding acceleration during reference sample collection12.

| Qiskit Instruction / Construct | Stim Target Equivalent | Semantic & Indexing Transformation Rules |
| --- | --- | --- |
| id, x, y, z, h, s, sdg, sx, sxdg | I, X, Y, Z, H, S, S_DAG, SQRT_X, SQRT_X_DAG | Direct 1-to-1 target mapping via flattened global integer qubit_map[q]17. |
| cx, cy, cz, swap, iswap | CX, CY, CZ, SWAP, ISWAP | Map control and target indices [q0, q1]; coalesce consecutive CX operations into a single batched CX q0 q1 q2 q3 call17. |
| Adjacent cx(c, t) + cz(c, t) | CY c t + phase fixup (S c) | Prevents the phase kickback error that corrupts stabilizer ancilla measurements18. |
| reset | R (or RZ, RX, RY) | Collapses target qubit to in tableau; pre-reset errors are ordered after the R instruction17. |
| measure(q, c) | M q (or MZ, MX, MY, MR) | Appends (creg_name, clbit_idx) to measurement_data list; updates latest pointer for rec[-k] lookup17. |
| barrier | TICK | Marks time-slice boundaries for circuit.diagram("timeslice-svg") and idle-noise insertion12. |
| x(q).c_if(clbit, 1) | CX rec[-k] q | Resolves clbit to relative offset 17. |
| ForLoopOp(range(N), body) | REPEAT N { body SHIFT_COORDS(0,0,1) } | Preserves compact loop structure; validates that measurement offsets inside the loop body are shift-invariant12. |

### Phase 2: Automated Custom Gate and Unitary Synthesis Engine (Weeks 4–7)

Phase 2 addresses the core laboratory requirement that user-defined custom quantum gates—whether specified as composite sub-circuits or raw  unitary matrices—be translated automatically without manual intervention25. When the transpiler encounters a custom qiskit.circuit.Gate or Instruction absent from the base mapping dictionary, it first inspects whether the object contains a composite .definition sub-circuit25. If a composite definition exists, the engine invokes Qiskit's HighLevelSynthesis pass to recursively unroll the gate while attaching a custom Stim instruction tag (inst_name[custom_gate_tag]) via stim.Circuit.append(..., tag=gate.name) so that downstream noise models and fault diagnostics retain visibility into the user's custom macro-gate boundaries25.

Before attempting matrix-to-Clifford conversion on opaque UnitaryGate matrices, the Phase 2 pipeline executes a two-qubit unitary block consolidation pass across any contiguous sequence of custom single- and two-qubit unitary matrices acting on the same qubit pair9. In silicon-spin circuits, this pass automatically multiplies sequences of non-Clifford fractional exchange pulses (such as paired  operations interleaved with single-qubit rotations) into their consolidated  unitary matrix 22. For every consolidated custom unitary matrix , the engine queries an LRU cache keyed by the rounded matrix byte-hash and executes the  coefficient magnitude and phase check followed by tableau = stim.Tableau.from_unitary_matrix(U, endian="little")19. The resulting tableau is decomposed into a native stim.Circuit via tableau.to_circuit(method="elimination") (or via Qiskit's CliffordUnitarySynthesis plugin when hardware coupling-map constraints on a Target must be respected)9. Every synthesized sub-circuit is automatically verified for exact algebraic equivalence by running stim.TableauSimulator.do_circuit(sub_circuit) and asserting simulator.current_inverse_tableau() ** -1 == tableau before remapping local indices [0..k-1] to the global circuit qubits30. If stim.Tableau.from_unitary_matrix raises a ValueError—confirming that the consolidated gate is genuinely non-Clifford57—the router automatically dispatches the circuit either to the Pauli-twirling approximator (for coherent error channels) or to tsim.Circuit / clifft for exact near-Clifford stabilizer-rank sampling17.

### Phase 3: Automated Detector Inference and UnityQ Hardware Noise Injection (Weeks 8–10)

A major source of manual rework when moving from Qiskit to Stim is that Qiskit circuits only specify gates and measurements, whereas Stim requires explicit DETECTOR and OBSERVABLE_INCLUDE declarations alongside inline Pauli noise instructions to construct a Detector Error Model12. Phase 3 automates both annotations. For engineers who specify detectors and logical observables declaratively in Qiskit (as lists of classical bits or dictionaries containing "clbits", "qubits", and "time" keys, matching qiskit-qec conventions), the converter resolves each classical bit to its relative StimTarget_rec offset and appends DETECTOR(x, y, t) and OBSERVABLE_INCLUDE(log_ind) instructions17. More powerfully, when a user supplies an unannotated Qiskit QEC circuit, the Phase 3 engine leverages Stim's stabilizer flow methods (stim.Circuit.flow_generators and stim.Circuit.solve_flow_measurements) alongside noiseless TableauSimulator tracking to automatically discover minimal measurement sets whose noiseless parity is deterministic across consecutive TICK or REPEAT cycles, emitting DETECTOR statements without human intervention16. Immediately after annotation, the engine enforces the completeness invariant64:

Any discrepancy alerts the user to missing or non-deterministic detectors prior to simulation34.

In parallel, Phase 3 implements a hardware-aware noise injector that translates either a qiskit_aer.noise.NoiseModel17 or a declarative UnityQNoiseProfile calibrated to Equal1's 22FDX CMOS spin-qubit benchmarks ( 1Q gate fidelity at 140 ns,  2Q gate fidelity at 200 ns, and  parity readout fidelity at 10 s) into inline Stim noise instructions3. Single- and two-qubit Pauli channels are mapped to PAULI_CHANNEL_1 and PAULI_CHANNEL_2 (or DEPOLARIZE1 and DEPOLARIZE2), reset errors are inserted immediately after R instructions, and measurement flip errors are inserted immediately before M instructions17. Crucially, using the TICK boundaries and physical gate durations, the noise compiler calculates the exact idling time of data qubits waiting during the 10 s parity readout window and automatically injects an idling Pauli error channel onto all unmeasured qubits, eliminating the need for manual idle_# placeholder gates3.

### Phase 4: Detector Error Model Extraction and PyMatching Decoder Bridge (Weeks 11–13)

Phase 4 automates the transition from the noisy stim.Circuit to the PyMatching decoder and supports both simulated Monte Carlo benchmarking and real QPU or Qiskit bitstring decoding14. The bridge compiles the Detector Error Model via dem = stim_circuit.detector_error_model(decompose_errors=True, approximate_disjoint_errors=True) and constructs the cached pymatching.Matching.from_detector_error_model(dem) decoder instance14. Because constructing the Matching object is often slower than the decoding step itself, the bridge caches the compiled graph across parameter sweeps66. If decompose_errors=True encounters irreducible hyperedges from non-CSS codes or custom multi-qubit gates, the bridge catches the decomposition error, logs the offending circuit fault coordinates via stim_circuit.explain_detector_error_model_errors(), and falls back to extracting hypergraph check matrices via detector_error_model_to_check_matrices(dem) for decoding with BeliefMatching or BpOsdDecoder35.

To support both simulation and experimental hardware testing, Phase 4 provides a dual-path execution harness. In bulk simulation mode, it compiles stim_circuit.compile_detector_sampler(), samples shots with separate_observables=True, executes C++ batch decoding via matcher.decode_batch(syndrome), and computes the logical error rate14. In hardware/Qiskit testing mode—used when measurement bitstrings are collected from Qiskit's simulator under time-dependent noise or from Equal1's physical UnityQ processor7—the bridge uses the noiseless annotated stim.Circuit to compile a measurement-to-detection-event converter via stim_circuit.compile_m2d_converter()16. It reverses Qiskit's little-endian bitstring formatting ([::-1]), converts the raw measurements into detection events and actual observable flips, and passes those detection events directly into matcher.decode_batch()16.

### Phase 5: Ecosystem Integration, Diagnostics, and Deployment (Weeks 14–16)

The final phase packages the transpiler and decoder bridge into an ergonomic SDK integrated into the Equal1 Timișoara laboratory's continuous integration and benchmarking workflows2. The library exposes both a Qiskit PassManager plugin and a functional API that returns the translated stim.Circuit, the stim.DetectorErrorModel, the pymatching.Matching graph (exportable to networkx.Graph via matcher.to_networkx()), and the decoded logical error rate report31. For large-scale threshold estimation across code distances and noise levels, Phase 5 wraps the pipeline into sinter.Task objects so that parallelized Monte Carlo simulations execute seamlessly via sinter.collect12. Furthermore, automated diagnostic hooks verify that len(stim_circuit.shortest_graphlike_error()) matches the target code distance  and generate interactive SVG time-slice (timeslice-svg) and 3D match-graph (matchgraph-3d) diagrams whenever a custom gate degrades the effective fault distance12.

## Reference Implementation Blueprint for the Conversion Core

To demonstrate how custom gate synthesis, measurement record translation, and PyMatching batch decoding operate cohesively in code, the following Python module blueprint implements the core transpilation logic required for Phases 1, 2, and 414.

Python

import numpy as np
import qiskit
from qiskit.quantum_info import Operator
import stim
import pymatching


class QiskitStimPyMatchingBridge:
    """Automated transpiler and syndrome decoding bridge for Qiskit, Stim, and PyMatching."""

    STANDARD_MAP = {
        "id": "I", "x": "X", "y": "Y", "z": "Z",
        "h": "H", "s": "S", "sdg": "S_DAG",
        "sx": "SQRT_X", "sxdg": "SQRT_X_DAG",
        "cx": "CX", "cy": "CY", "cz": "CZ",
        "swap": "SWAP", "iswap": "ISWAP",
        "reset": "R", "barrier": "TICK",
    }

    def __init__(self, gate_depol_1q=0.001, gate_depol_2q=0.007, readout_flip=0.01):
        # Default noise parameters calibrated to Equal1 UnityQ 22FDX benchmarks
        # (99.9% 1Q fidelity, 99.3% 2Q fidelity, 99.0% parity readout fidelity)
        self.gate_depol_1q = gate_depol_1q
        self.gate_depol_2q = gate_depol_2q
        self.readout_flip = readout_flip
        self._unitary_cache = {}

    def synthesize_custom_unitary(self, matrix, target_qubits):
        """Automatically converts an arbitrary k-qubit unitary matrix into a stim.Circuit."""
        key = np.round(matrix, decimals=10).tobytes()
        if key not in self._unitary_cache:
            try:
                # Extract stabilizer tableau using Qiskit's little-endian convention
                tableau = stim.Tableau.from_unitary_matrix(matrix, endian="little")
            except ValueError as exc:
                raise ValueError(
                    "Custom gate matrix is non-Clifford after block consolidation. "
                    "Enable mode='near_clifford_exact' (Tsim/Clifft) or 'pauli_twirl_qec'."
                ) from exc
            # Synthesize elementary Clifford circuit via Gaussian elimination
            local_circuit = tableau.to_circuit(method="elimination")
            self._unitary_cache[key] = local_circuit

        local_circuit = self._unitary_cache[key]
        remapped = stim.Circuit()
        for inst in local_circuit:
            mapped_targets = [target_qubits[t.value] for t in inst.targets_copy()]
            remapped.append(inst.name, mapped_targets, inst.gate_args_copy())
        return remapped

    def transpile_to_stim(self, qc, detectors=None, observables=None, inject_noise=True):
        """Translates a Qiskit QuantumCircuit (including custom gates) into a stim.Circuit."""
        qubit_map = {q: idx for idx, q in enumerate(qc.qubits)}
        clbit_map = {c: idx for idx, c in enumerate(qc.clbits)}
        # Track chronological measurement record indices for each classical bit
        meas_record_history = []
        latest_clbit_meas_idx = {}

        stim_circuit = stim.Circuit()

        for instruction in qc.data:
            op = instruction.operation
            name = op.name.lower()
            q_indices = [qubit_map[q] for q in instruction.qubits]
            c_indices = [clbit_map[c] for c in instruction.clbits]

            # 1. Handle measurements and record tracking
            if name == "measure":
                if inject_noise and self.readout_flip > 0:
                    stim_circuit.append("X_ERROR", q_indices, self.readout_flip)
                stim_circuit.append("M", q_indices)
                for c_idx in c_indices:
                    meas_pos = len(meas_record_history)
                    meas_record_history.append(c_idx)
                    latest_clbit_meas_idx[c_idx] = meas_pos
                continue

            # 2. Handle standard Clifford gates, resets, and barriers
            if name in self.STANDARD_MAP:
                stim_op = self.STANDARD_MAP[name]
                stim_circuit.append(stim_op, q_indices)
                if inject_noise:
                    if name == "reset" and self.readout_flip > 0:
                        stim_circuit.append("X_ERROR", q_indices, self.readout_flip)
                    elif len(q_indices) == 1 and name not in ("id", "reset", "barrier"):
                        if self.gate_depol_1q > 0:
                            stim_circuit.append("DEPOLARIZE1", q_indices, self.gate_depol_1q)
                    elif len(q_indices) == 2 and self.gate_depol_2q > 0:
                        stim_circuit.append("DEPOLARIZE2", q_indices, self.gate_depol_2q)
                continue

            # 3. Handle custom composite gates or arbitrary custom unitary matrices
            matrix = Operator(op).data
            synthesized_subcircuit = self.synthesize_custom_unitary(matrix, q_indices)
            stim_circuit += synthesized_subcircuit
            if inject_noise:
                if len(q_indices) == 1 and self.gate_depol_1q > 0:
                    stim_circuit.append("DEPOLARIZE1", q_indices, self.gate_depol_1q)
                elif len(q_indices) == 2 and self.gate_depol_2q > 0:
                    stim_circuit.append("DEPOLARIZE2", q_indices, self.gate_depol_2q)

        total_meas = len(meas_record_history)

        # 4. Append DETECTOR annotations using relative measurement record offsets
        if detectors:
            for det in detectors:
                rec_targets = [
                    stim.target_rec(latest_clbit_meas_idx[c_idx] - total_meas)
                    for c_idx in det["clbits"]
                ]
                coords = det.get("coords", [])
                stim_circuit.append("DETECTOR", rec_targets, coords)

        # 5. Append OBSERVABLE_INCLUDE annotations
        if observables:
            for obs_idx, clbit_list in enumerate(observables):
                rec_targets = [
                    stim.target_rec(latest_clbit_meas_idx[c_idx] - total_meas)
                    for c_idx in clbit_list
                ]
                stim_circuit.append("OBSERVABLE_INCLUDE", rec_targets, obs_idx)

        return stim_circuit

    def benchmark_with_pymatching(self, stim_circuit, num_shots=100_000):
        """Extracts the Detector Error Model, builds the PyMatching graph, and decodes batches."""
        dem = stim_circuit.detector_error_model(
            decompose_errors=True,
            approximate_disjoint_errors=True
        )
        matcher = pymatching.Matching.from_detector_error_model(dem)
        sampler = stim_circuit.compile_detector_sampler()
        syndrome, actual_observables = sampler.sample(
            shots=num_shots,
            separate_observables=True
        )
        predicted_observables = matcher.decode_batch(syndrome)
        num_errors = int(np.sum(np.any(predicted_observables != actual_observables, axis=1)))
        return {
            "shots": num_shots,
            "logical_errors": num_errors,
            "logical_error_rate": num_errors / num_shots,
            "num_detectors": matcher.num_detectors,
            "num_edges": matcher.num_edges,
        }

## Strategic Conclusions and Implementation Recommendations

Executing this five-phase engineering plan transforms a fragmented, error-prone manual workflow into a deterministic compiler pipeline tailored for silicon-spin fault-tolerant research3. First, prioritizing two-qubit unitary block consolidation prior to Clifford verification is essential for silicon-spin processors: because native Heisenberg exchange pulses such as  are individually non-Clifford yet compose into Clifford CX, CZ, or iSWAP gates, fusing contiguous custom gates into consolidated  matrices before invoking stim.Tableau.from_unitary_matrix(..., endian="little") prevents false rejections on hardware-native circuits19. Second, adopting the annotated stim.Circuit and its stim.DetectorErrorModel as the canonical Intermediate Representation unifies both simulation and hardware decoding: PyMatching loads its matching graph directly from the DEM via pymatching.Matching.from_detector_error_model, while stim.Circuit.compile_m2d_converter() translates raw bitstrings from Qiskit simulators or physical Bell-1/UnityQ runs into PyMatching detection events without custom glue code14.

Finally, embedding automated algebraic invariants at every stage—specifically round-trip tableau verification (Tableau.from_circuit(synthesized) == Tableau.from_unitary_matrix(U)) for custom gates, detector completeness (num_detectors + num_observables == count_determined_measurements()), and graphlike distance verification (len(shortest_graphlike_error()) == d)—eliminates silent endianness, phase-kickback, and classical indexing bugs18. Once deployed in the Timișoara laboratory, this unified toolchain will not only accelerate software QEC benchmarking across Qiskit, Stim, and PyMatching, but also provide an automated compiler frontend capable of exporting exact parity-check matrices and discretized integer edge weights directly to the cryogenic ARM Cortex decoding cores operating at 300 mK on the UnityQ SoC2.

#### Works cited

- Company - Equal1, https://www.equal1.com/company

- Software Stack / Framework Engineer - Equal1 | BeBee, https://bebee.com/ro/jobs/software-stack-framework-engineer-equal1-timisoara--ss-ro-1kz8jj8

- UnityQ - Equal1, https://www.equal1.com/technology

- Kvantify and Equal1 Announce Strategic Quantum Alliance, https://www.equal1.com/insights/kvantify-and-equal1-announce-strategic-quantum-alliance

- Si-based quantum dots for spin qubits pushes forward | Electronics360, https://electronics360.globalspec.com/article/22212/si-based-quantum-dots-for-spin-qubits-pushes-forward

- ESA Brings Quantum Computing System to Earth Observation, https://thequantuminsider.com/2026/07/16/esas-first-quantum-computer-will-shift-computing-frontiers-in-space/

- Top Silicon Spin Quantum Computing Companies 2026, https://quantumzeitgeist.com/top-silicon-spin-quantum-computing-companies/

- Creating Entangled Logical Qubits in the Heavy-Hex Lattice with, https://link.aps.org/doi/10.1103/PRXQuantum.5.040334

- CliffordUnitarySynthesis (v2.1) | IBM Quantum Documentation, https://quantum.cloud.ibm.com/docs/api/qiskit/2.1/qiskit.transpiler.passes.synthesis.clifford_unitary_synth_plugin.CliffordUnitarySynthesis

- calculating the unitary of a circuit using Qiskit's simulator, https://quantumcomputing.stackexchange.com/questions/29283/calculating-the-unitary-of-a-circuit-using-qiskits-simulator

- Equal1 and Q-CTRL Integrate Autonomous Calibration for Silicon, https://quantumcomputingreport.com/equal1-and-q-ctrl-integrate-autonomous-calibration-for-silicon-quantum-systems/

- Tutorial - Estimating the Surface Code Threshold - NordIQuEst -, https://nordiquest.net/application-library/training-material/qas2024/notebooks/surface_code_threshold.html

- Clifford Circuits: Simulating Thousands of Qubits in Milliseconds with, https://zksf.org/blog/clifford-circuits-stim/

- PyMatching 2 — PyMatching 2.1.0 documentation, https://pymatching.readthedocs.io/

- Saadatmand, S. N.; Wilson, Tyler L.; Hodson, Mark J., https://research.aalto.fi/files/231327373/Superconducting_qubits_in_the_millions_The_potential_and_limitations_of_modularity.pdf

- How can I use surface code measurement data produced with Qiskit, https://quantumcomputing.stackexchange.com/questions/35673/how-can-i-use-surface-code-measurement-data-produced-with-qiskit-to-run-matching

- qiskit_qec.utils.stim_tools - Qiskit QEC 0.0.0, https://qiskit-community.github.io/qiskit-qec/_modules/qiskit_qec/utils/stim_tools.html

- How to interface stim to a generic qiskit decoder?, https://quantumcomputing.stackexchange.com/questions/28715/how-to-interface-stim-to-a-generic-qiskit-decoder

- Stim v1.12 Python API Reference - GitHub, https://github.com/quantumlib/Stim/wiki/Stim-v1.12-Python-API-Reference

- SynDebug: A Framework to Debug Logic Bugs in Fault-Tolerant, https://arxiv.org/html/2610.22275v1

- How silicon spin qubits work - Anyon Computing, https://www.anyoncomputing.com/post/how-silicon-spin-qubits-work

- Silicon Spin Qubits: The CMOS Path to Quantum Computing, https://postquantum.com/quantum-modalities/silicon-based-qubits/

- Simulation | Deltakit v0.5.1, https://deltakit-docs.riverlane.com/en/docs/guide/simulation.html

- Simulating surface codes using stim - Deltakit Textbook - Riverlane, https://textbook.riverlane.com/en/latest/notebooks/ch5-decoding-surfcodes/simulating-surface-codes-stim.html

- qiskit/qiskit/transpiler/passes/synthesis/high_level_synthesis.py at, https://github.com/Qiskit/qiskit/blob/main/qiskit/transpiler/passes/synthesis/high_level_synthesis.py

- PyMatching - Release 2.1.dev1 Oscar Higgott and Craig Gidney, https://pymatching.readthedocs.io/_/downloads/en/latest/pdf/

- Models - Qibo · v0.3.5, https://qibo.science/qibo/stable/api-reference/qibo.html

- How do I check if a gate represented by Unitary U is a Clifford Gate?, https://quantumcomputing.stackexchange.com/questions/13157/how-do-i-check-if-a-gate-represented-by-unitary-u-is-a-clifford-gate

- Stim: a fast stabilizer circuit simulator - arXiv, https://arxiv.org/pdf/2103.02202

- how to go from matrix to tableau to circuit in qiskit or stim, https://quantumcomputing.stackexchange.com/questions/27326/how-to-go-from-matrix-to-tableau-to-circuit-in-qiskit-or-stim

- Python API Documentation - PyMatching - Read the Docs, https://pymatching.readthedocs.io/en/latest/api.html

- How to force PyMatching into the opposite equivalence class, https://quantumcomputing.stackexchange.com/questions/35572/how-to-force-pymatching-into-the-opposite-equivalence-class

- Tutorial - Fault-Tolerant Quantum Computing with CSS codes, https://nordiquest.net/application-library/training-material/qas2024/notebooks/css_code_steane.html

- How to implement a controlled gate in Stim with the following control, https://quantumcomputing.stackexchange.com/questions/37084/how-to-implement-a-controlled-gate-in-stim-with-the-following-control-logic

- Stim, Surface Code and Parity check matrix · Issue #73 - GitHub, https://github.com/oscarhiggott/PyMatching/issues/73

- error correction - How is the match graph extracted from a stim.circuit, https://quantumcomputing.stackexchange.com/questions/30062/how-is-the-match-graph-extracted-from-a-stim-circuit-detector-error-model

- Quantum Error Correction Fundamentals | Nelop Systems, https://nelop.com/knowledge.html?article=quantum/quantum-error-correction

- Quantum Simulation of Si/SiGe Devices with Experimentally ... - arXiv, https://arxiv.org/html/2609.27724v1

- CMOS Position-Based Charge Qubits - University College Dublin, https://www.ucd.ie/nanoelectronics/t4media/IEEE_Access_8_4182_(2019).pdf

- Introducing Tsim: Fast, Non-Clifford Circuit Simulation for QEC, https://www.quera.com/blog-posts/introducing-tsim-fast-non-clifford-circuit-simulation-for-qec-researchers

- Are there qiskit or stim examples of how to decode a subsystem code, https://quantumcomputing.stackexchange.com/questions/32760/are-there-qiskit-or-stim-examples-of-how-to-decode-a-subsystem-code

- スタビライザーシミュレータのStimを使ってみる【量子計算】, https://trsasasusu.com/blog/168/stim-quantum-surface-code/

- in stim how do you get a detector check and logical observable, https://quantumcomputing.stackexchange.com/questions/40823/in-stim-how-do-you-get-a-detector-check-and-logical-observable-matrices-from-tab

- Retargetable Compilation for Fault-Tolerant Quantum Computing, https://arxiv.org/pdf/2609.13388

- Frequent 'stim' Questions - Quantum Computing Stack Exchange, https://quantumcomputing.stackexchange.com/questions/tagged/stim?tab=Frequent

- Stim/doc/getting_started.ipynb at main · quantumlib/Stim - GitHub, https://github.com/quantumlib/Stim/blob/main/doc/getting_started.ipynb

- qiskit_qec.utils.get_stim_circuits - Qiskit QEC 0.0.0 - GitHub Pages, https://qiskit-community.github.io/qiskit-qec/stubs/qiskit_qec.utils.get_stim_circuits.html

- HighLevelSynthesis (latest version) | IBM Quantum Documentation, https://quantum.cloud.ibm.com/docs/api/qiskit/qiskit.transpiler.passes.HighLevelSynthesis

- Releases · quantumlib/Stim - GitHub, https://github.com/quantumlib/Stim/releases

- unknown_url

- A Visualisation and Analysis Toolkit for Universal ... - QIT at UNIUD, https://qit.uniud.it/publications/visual_toolkit.pdf

- Introducing Clifft: Exact Simulation for Early Fault-Tolerant Circuits, https://unitary.foundation/posts/2026_clifft

- Clifft, https://unitaryfoundation.github.io/clifft/stable/

- Stim/doc/python_api_reference_vDev.md at main - GitHub, https://github.com/quantumlib/Stim/blob/main/doc/python_api_reference_vDev.md

- MITM Search for T-Gate Quantum Computing - BlueQubit | App, https://app.bluequbit.io/tutorial/t-count-mitm

- How to convert Tableau to Circuit in stim, https://quantumcomputing.stackexchange.com/questions/27642/how-to-convert-tableau-to-circuit-in-stim

- Stim v1.13 Python API Reference · quantumlib/Stim Wiki - GitHub, https://github.com/quantumlib/Stim/wiki/Stim-v1.13-Python-API-Reference

- BeliefMatching/src/beliefmatching/belief_matching.py at main - GitHub, https://github.com/oscarhiggott/BeliefMatching/blob/main/src/beliefmatching/belief_matching.py

- Why I am continuously getting "failed to decompose hyperedges into, https://quantumcomputing.stackexchange.com/questions/37375/why-i-am-continuously-getting-failed-to-decompose-hyperedges-into-edges-in-sti

- How can we decode non CSS codes like XZZX code with Pymatching?, https://quantumcomputing.stackexchange.com/questions/35788/how-can-we-decode-non-css-codes-like-xzzx-code-with-pymatching

- How to convert stim circuit to a detector check matrix and a logical, https://quantumcomputing.stackexchange.com/questions/37710/how-to-convert-stim-circuit-to-a-detector-check-matrix-and-a-logical-observable

- How to get parity check matrix from a circuit in stim, https://quantumcomputing.stackexchange.com/questions/33657/how-to-get-parity-check-matrix-from-a-circuit-in-stim

- beliefmatching - PyPI, https://pypi.org/project/beliefmatching/

- Using Pymatching to decode lattice surgery process, https://quantumcomputing.stackexchange.com/questions/33700/using-pymatching-to-decode-lattice-surgery-process

- Emulating repetition code data subsampling, https://quantumcomputing.stackexchange.com/questions/33299/emulating-repetition-code-data-subsampling

- Toric code example — PyMatching 2.1.dev1 documentation, https://pymatching.readthedocs.io/en/latest/toric-code-example.html